Video Optical Transceiver Series

1/4

4-Channel Video + 1-Channel reverse RS485 data                           Optical Transceiver

Product Overview

The TCE-SF4V1FD-S20T/R series digital video optical transceiver has a full-featured all-digital video optical transmission equipment that can transmit 4-Channel high-quality (10Bit) uncompressed video information and 1-Channel control signal on one fiber at the same time, optional audio、 data、 switch volume . This product is convenient for field wiring, no need for any photoelectric adjustment; with power, optical and data loop instructions; optional standalone desktop installation and modular rack-mounted two structural forms. This series digital optical products are mainly used for long-distance optical fiber transmission monitoring system, such as security monitoring, highways, electronic police, automation, intelligent community, customs, electricity, water, petroleum, chemical and many other fields.

Optical Characteristics：

| fiber type/ parameter | Fiber interface type | Emission wavelength nm | Transmit power dBm | Receiver sensitivity dBm | Light saturation dBm | Optical loss dBm/km | Transmission distance km |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi Mode | SC/FC | 850 | -18.5~-14 | -35 | -8 | 2 | 0~2 |
|  | SC/FC | 1310 | -19.5~-16 | -36 | -9 | 1 | 0~3 |
| Single Mode | SC/FC | 1310 | -15~-10 | -34 | -8 | 0.5 | 0~20 |
|  | SC/FC | 1310 | -12~-5 | -36 | -3 | 0.5 | 0~40 |
|  | SC/FC | 1550 | -5~0 | -38 | 2 | 0.25 | 0~60 |

Produc

Video Optical Transceiver Series

2/4

t Features：

- 10-bit digital encoding and uncompressed video transmission;

- 5Hz-10MHz video channel; automatically compatible with PAL, NTSC, SECAM video system; with APC circuit, the output optical power is constant, the dynamic range is large;

- Without downtime maintenance in the process of using, can be hot-swappable; Gigabit fiber transmission, large capacity, easy to upgrade;

- Power and other parameters status indicator LED can monitor the operation status of the system;

- Support any high resolution video signal; support video lossless regenerative relay;

- Support RS232/RS485/RS422 or Manchester code;

- Advanced adaptive technology, does not require on-site electrical or optical adjustment;industrial modular design makes the device reliable and flexible;

- Can automatically restore the overload protection; unique full built-in power supply design;

- Desktop, wall mounted, 19 inch 1U rack or 4U card insert; support for user customization.

Parameter

Application environment:

security system, TV medical, high-fidelity video and video conference call, real-time monitoring and control system, intelligent traffic monitoring system (ITS), remote multimedia teaching / campus monitoring, long-distance radio and television transmission system

Optical characteristics:

Optical interface: SC/FC/ST

Wavelength: single mode 1310nm/1550nm ；multimode 850nm/1310nm

Fiber; multimode, single mode;

Transmission power transmission: -9dBm/-5dBm

Transmission length: 0-100km;

Receiving sensitivity: better than -36dBm

Features: Can be customized according to user needs for different interface types.

- Video characteristics:

- Channel：4-Channel video

- Video interface form: BNC

- Signal system: PAL/NTSC/SECAM; Nominal input/output level: 1Vp-p

Video Optical Transceiver Series

3/4

Video input/output impedance: 75Ω（unbalanced)

- Video sampling: 15MHz high speed sampling

- Video input/output voltage: 1Vp-p

- Video bandwidth: 8MHz

- Differential gain:(10% -90% APL) DG<1% (typical)

- Differential phase:(10% -90% APL) DP<0.7o (typical)

- Video signal to noise ratio(weighted):S/N ≥ 70dB (maximum optical link loss)

- Color brightness delay difference of 10ns (typical)

- Color brightness gain difference: ± 10% (typical);

- Note: support forward, reverse, BIDI video.

Data characteristics:

Data Channel：1-Channel reverse data

Interface type:  RS232/RS485/RS422/ Manchester code

- Interface signal: RS-232, RS-422, RS-485, Man code / Biphase, switch signal;

- Interface features：meet the ITU-T V.24 standard

- Connection mode:DCE

- Code rate:0～256Kbps

Bit Error Rate：≤10-9

- Note: support forward, reverse, BIDI data.

Audio characteristics:

Audio channels: 1-Channel Bidirectional audio (optional)

Each channel bandwidth: 20Hz ~ 20KHz; Sampling frequency: 96KHz

Quantification level: 24bit;

Input impedance (Line-in): 47KΩ / unbalanced

Output impedance (Line-out): 10KΩ unbalanced

Nominal input and output level: 2Vp-p; Total harmonic distortion: 0.1% (typical)

Unbalanced Weighted Signal to Noise Ratio: 75dB (typical)

Switch interface parameters(RJ45/terminal):

Response time: <0.5ms  Physical interface: RJ45

Switch interface: Optical MOS relay 350V/120mA

Telephone Interface:FXO / FXS Interface Parameters (RJ45 / RJ11)

FXS phone port

Ringing voltage: 75V; Ringing frequency: 25HZ

Second-line input impedance: 600Ω (off-hook); Return loss: 40 dB

FXO switch interface;

Ring detection voltage: 35V; Ringing detection frequency: 17HZ-60HZ;

Second-line input impedance: 600Ω (off-hook); Return loss: 40 dB

Features: Optional RS232 / RS485 / RS422 different types of data interface, or both.

Video Optical Transceiver Series

4/4

Ambient temperature: -30 ℃ ~ 75 ℃;

Storage temperature: -40 ℃ ~ 85 ℃

Relative humidity: 0 ~ 95% (non-Condensing);

Size: Standalone (225mm*200mm*40mm)

Size: Card (standard 4U centralized management rack)

Rated supply voltage: AC220V / 50Hz;

Bit error rate: ≤10-9

Internal power consumption: 2.5 W;

MTBF: ≥105 hours

Application Drawing