Video Optical Transceiver Series

1/3

24-Channel Video + 1-Channel reverse RS485 data                           Optical Transceiver

Product Overview

The TCE-SFU24V1FD-S20T/R series digital video optical transceiver has a full-featured all-digital video optical transmission equipment that can transmit 24-Channel high-quality (8 Bit/10 Bit) uncompressed video information and 1-Channel reverse data on one fiber at the same time, optional audio、 data、 switch volume . This product is convenient for field wiring, no need for any photoelectric adjustment; with power, optical and data loop instructions; optional two structural forms:standalone desktop installation and modular rack-mounted . This series digital optical products are mainly used for long-distance optical fiber transmission monitoring system, such as security monitoring, highways, electronic police, automation, intelligent community, customs, electricity, water, petroleum, chemical and many other fields.

Product Features：

- 8/10-bit digital encoding and uncompressed video transmission;

- support any high-resolution video signal;

- 5Hz-10MHz video channel; automatically compatible with PAL, NTSC, SECAM video system with APC circuit, the output optical power is constant, the dynamic range is large;

- Without downtime maintenance in the process of using, can be hot-swappable; Gigabit fiber transmission, large capacity, easy to upgrade;

- Power and other parameters status indicator LED can monitor the operation status of the system;

- Support video lossless regenerative relay;

- Advanced adaptive technology, does not require on-site electrical or optical adjustment;industrial modular design makes the device reliable and flexible;

- Can automatically restore the overload protection; unique full built-in power supply design;

Video Optical Transceiver Series

2/3

Application environment:

security system, TV medical, high-fidelity video and video conference call, real-time monitoring and control system, intelligent traffic monitoring system (ITS), remote multimedia teaching / campus monitoring, long-distance radio and television transmission system.

Parameter

| Optical characteristics |  |  |  |
| --- | --- | --- | --- |
| Optical interface |  | Conventional FC (optional SC, ST) |  |
| Wavelength |  | single mode 1310nm/1550nm ；multimode 850nm/1310nm |  |
| Fiber |  | multimode, single mode |  |
| Transmission power |  | -9dBm/-5dBm |  |
| Transmission length |  | 0-100km |  |
| Receiving sensitivity |  | better than -36dBm |  |
| Video characteristics(V) |  |  |  |
| Number of Video interface |  | 24 video (optional 2-Channel , 4-Channel , 8-Channel , 16~128-Channel forward audio or BIDI audio) |  |
| Video interface form |  | BNC |  |
| Video input/output impedance |  | 75Ω（unbalanced) |  |
| Video input/output voltage |  | 1Vp-p |  |
| Video bandwidth |  | 8MHz |  |
| Video sampling |  | 15MHz high speed sampling |  |
| Differential gain |  | (10% -90% APL) DG<1% (typical) |  |
| Differential phase |  | (10% -90% APL) DP<0.7o (typical) |  |
| Video signal to noise ratio (weighted) |  | S/N ≥ 70dB (maximum optical link loss) |  |
| Data characteristics(D):(Optional) |  |  |  |
| Number of Data Channel |  | 1-Channel reverse data(Optional forward data, bidirectional data) |  |
| Data Interface Terminals |  | Standard Industrial Terminals or RJ45 |  |
| Interface signal |  | RS-232, RS-422, RS-485, Man code / Biphase, switch signal; |  |
| Interface features |  | meet the ITU-T V.24 standard |  |
| Connection mode |  | DCE |  |
| Code rate |  | 0～256Kbps |  |
| Bit Error Rate |  | ≤10-9 |  |
| Audio characteristics(A):(Optional) |  |  |  |
| Audio channels |  | 1-Channel Bidirectional audio (optional forward audio ,reverse audio) |  |
| Video Optical Transceiver Series 3/3 Each channel bandwidth |  | 20Hz ~ 20KHz |  |
| Sampling frequency |  | 96KHz |  |
| Quantification level |  | 24bit |  |
| Input impedance (Line-in) |  | 47KΩ / unbalanced |  |
| Output impedance (Line-out) |  | 10KΩ unbalanced |  |
| Nominal input and output level |  | 2Vp-p |  |
| Total harmonic distortion |  | 0.1% (typical) |  |
| Unbalanced Weighted Signal to Noise Ratio |  | 75dB (typical) |  |
| Switch interface parameters(K):(Optional) |  |  |  |
| Number of Switch channels |  | forward switch, bidirectional switch. |  |
| Response time |  | < 0.5ms |  |
| Switch interface |  | Optical MOS relay 350V/120mA |  |
| Physical interface |  | Standard Industrial Terminals |  |
| Ethernet characteristics (E):(Optional) |  |  |  |
| Rate |  | 10/100M Adaptive |  |
| Protocol: |  | Support IEEE802.3, IEEE802.1Q (VLAN) |  |
| Physical Interface |  | RJ45 , Auto-MDIX |  |
| Working mode |  | Full duplex / half duplex adaptive |  |
| Telephone characteristics (p):(Optional) |  |  |  |
| FXS phone port | Ringing voltage |  | 75V |
|  | Ringing frequency |  | 25HZ |
|  | Second-line input impedance |  | 600Ω (off-hook) |
|  | Return loss |  | 40 dB |
| General Specifications |  |  |  |
| Ambient temperature |  |  | -30 ℃ ~ 75 ℃ |
| Storage temperature |  |  | -40 ℃ ~ 85 ℃ |
| Relative humidity |  |  | 0 ~ 95% (non-Condensing) |
| Size |  |  | Independent rack (485mm*198mm*45mm) |
| Rated supply voltage |  |  | AC220V / 50Hz |
| Bit error rate |  |  | ≤10-9 |
| Internal power consumption |  |  | 1.5 W |
| MTBF |  |  | ≥105 hours |