Video Optical Transceiver Series

1/3

32-Channel Video + 1-Channel reverse RS485 data                           Optical Transceiver

Product Overview

The TCE-SFU32V1FD-S20T/R series digital video optical transceiver has a full-featured all-digital video optical transmission equipment that can transmit 32-Channel high-quality (8 Bit/10 Bit) uncompressed video information and 1-Channel reverse data on one fiber at the same time, optional audio、 data、 switch volume、telephone . This product is convenient for field wiring, no need for any photoelectric adjustment; with power, optical and data loop instructions; optional two structural forms:standalone desktop installation and modular rack-mounted . This series digital optical products are mainly used for long-distance optical fiber transmission monitoring system, such as security monitoring, highways, electronic police, automation, intelligent community, customs, electricity, water, petroleum, chemical and many other fields.

Product Features：

- ■ Monitoring terminal can be used standard 19 inch 1U chassis, remote using desktop or rack;

- ■ Remote optical transceiver can be hung on the wall;

- ■ Single-mode single-fiber transmission of video, audio / speech channel, data, switch and other signals;

- ■ Digital uncompressed optical fiber transmission;

- ■ Compatible with PAL, NTSC, SECAM and other standard video transmission;

- ■ Half-duplex / full-duplex data communication;

- ■ BIDI audio or speech;

- ■ Audio / Telephone, Ethernet is optional card (optional interface), user-friendly selection;

- ■ Industrial design, high reliability;

- ■ Full surface mount technology;

- ■ MTBF ≥ 80000 hours

Video Optical Transceiver Series

1/3

Application environment:

security system, TV medical, high-fidelity video and video conference call, real-time monitoring and control system, intelligent traffic monitoring system (ITS), remote multimedia teaching / campus monitoring, long-distance radio and television transmission system.

Parameter

| Optical characteristics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Optical interface |  | Conventional FC (optional SC, ST) |  |  |
| Wavelength |  | single mode 1310nm/1550nm ；multimode 850nm/1310nm |  |  |
| Fiber |  | multimode, single mode |  |  |
| Transmission power |  | -9dBm/-5dBm |  |  |
| Transmission length |  | 20km (basic type) (40km, 60km, 80km optional) |  |  |
| Receiving sensitivity |  | better than -36dBm |  |  |
| Video characteristics(V) |  |  |  |  |
| Number of Video interface |  | 32 video |  |  |
| Video interface form |  | BNC |  |  |
| Video input/output impedance |  | 75Ω（unbalanced) |  |  |
| Video input/output voltage |  | 1Vp-p |  |  |
| Differential gain |  | <1% (typical) |  |  |
| Differential phase |  | <1 ° (typical) |  |  |
| Video signal to noise ratio (weighted) |  | S/N ≥ 70dB (maximum optical link loss) |  |  |
| Video bandwidth |  | 2Hz～8MHz |  |  |
| 窗体顶端 Sampling frequency |  | 16MHz |  |  |
| Quantization level |  | 8/10bit |  |  |
| Signal System |  | PAL/NTSC/SECAM |  |  |
| Color brightness delay difference |  | 10ns |  |  |
| Color brightness gain difference |  | +/- 10% |  |  |
| Data characteristics(D):(Optional) |  |  |  |  |
| Data Interface Terminals |  | Standard Industrial Terminals |  |  |
| Data format |  | RS-232 / RS-485 optional |  |  |
| Work mode |  | half duplex / full duplex optional |  |  |
| Transmission rate |  | 0～115.2Kbps |  |  |
| Bit Error Rate |  | ≤10-9 |  |  |
| Audio characteristics(A):(Optional) |  |  |  |  |
| input/output voltage |  | 1Vp-p |  |  |
| input/Output impedance |  | 600Ω, unbalanced |  |  |
| Video Optical Transceiver Series 3/3 Audio bandwidth |  | 10Hz ~ 20KHz |  |  |
| Sampling frequency |  | 8KHz |  |  |
| Switch interface parameters(K):(Optional) |  |  |  |  |
| Physical interface |  | Standard Industrial Terminals |  |  |
| Input signal |  | any active or passive alarm, switch signal input, support TTL, or passive switch, button |  |  |
| Output signal |  | any active or passive alarm, switch signal output, support for TTL, or relay contact output |  |  |
| Note: the switch can only access 5V signal or passive switch, the output is used to drive an external controller, and then control the high-voltage equipment |  |  |  |  |
| Telephone characteristics (p):(Optional) |  |  |  |  |
| Interface Type |  | Automatic (FXS), exchange (FXO) |  |  |
| Sampling frequency |  | 8KHz |  |  |
| Coding |  | A law |  |  |
| Bit rate |  | 64Kbit / s |  |  |
| Maximum loop resistance |  | 2000Ω |  |  |
| Effective transmission bandwidth |  | 300-3400HZ / 600Ω |  |  |
| Idle channel noise |  | <-67dB |  |  |
| Ring current voltage |  | 25 ± 3Hz, 75 ± 15V |  |  |
| Net attenuation frequency |  | to meet the ITU-T recommendations |  |  |
| Free talk noise |  | no more than -65dBm0p |  |  |
| Road cross-talk |  | no more than -65dBm0 |  |  |
| Amplitude characteristics |  | Meet ITU-T recommendations |  |  |
| Total distortion |  | Meet ITU-T recommendations |  |  |
| Power supply conditions |  |  |  |  |
| -48VDC |  |  | Power supply range -36V ~ -72V, ripple ≤ 240mVp-p |  |
| 220VAC |  |  | Power supply range 110V ~ 265V, 50Hz |  |
| Power consumption |  |  | The whole is less than 3W |  |
| Environmental requirements |  |  |  |  |
| working environment: | Ambient temperature |  |  | -10 ℃ ~ 55 ℃ |
|  | Relative humidity |  |  | ≤ 90% (non-condensing) |
|  | Atmospheric pressure |  |  | 86 ~ 106KPa |
| Storage environment: | Ambient temperature |  |  | -20 ~ 80 ℃ |
|  | Relative humidity |  |  | ≤95% |
| Size |  |  |  | 230x160x43mm |