Fiber Media Converter Series

1/2

10/100M single port Mini Fiber Media Converter

Product Description

The TBC-MC3302S series of mini (micro) Ethernet fiber optic transceivers, the size is only 67mm * 75mm * 26mm, smaller than the business card, easy to install in a variety of waterproof box or integrated ball machine inside (fiber Ball) and explosion-proof cameras, the product can be stable in the 0 °C ~ +50 °C harsh climate environment, +9 ~ +36 V wide range of power supply design can adapt to many occasions. This series can be a single fiber transmission 1 channel Ethernet , support a video server or network camera access.

This  Ethernet optical transceiver (fiber optic transceivers) with Ethernet interface triple lightning protection design, effective against lightning damage.

Product Features

- Mini, size only 67mm * 75mm * 26mm

- Dual / single fiber transmission 1 channel Ethernet, effectively saving fiber

- 10/100M adaptive

- Fully support IEEE802.3、IEEE802.3u、IEEE802.3ab、IEEE802.3z

- support IEEE802.3X Full duplex flow control and BACKPRESSURE half duplex flow control

- Detects the VLAN address resolution protocol for packets

- The network port supports parallel / crossover cable adaptation

- Single / dual fiber optional, single mode / multi-mode fiber transmission, no relay Maximum transmission distance 100KM

- No electromagnetic interference (EMI), radio frequency interference (RFI) and common ground circuit interference

- With power, optical, Ethernet status LED instructions, you can monitor the system running

- Industrial design, SMT surface mount technology

- Iron shell Miniature structure

- Low power consumption, plug and play, support hot-swappable, debug-free installation

- + 9 ~ 36V wide range of power supply design

- Ethernet Interface triple lightning protection design

Fiber Media Converter Series

2/2

Product Specification

| Fiber interface parameters (single fiber / dual fiber) |  |
| --- | --- |
| Wavelength | Single mode 1310nm/1550nm； Multimode 850nm/1310nm |
| Number of fiber cores | Duplex , Simplex |
| Transmission power | -9dBm/-5dBm |
| Transmission distance | 0-100KM (dual fiber) 0-120 KM |
| Receiver sensitivity | better than -34dBm |
| Fiber Connector | FC/SC/ST/LC optional |
| Ethernet interface |  |
| Number of channels | 1channel |
| Rate | 10M,100M |
| Standard | IEEE802.3/ IEEE802.3u / IEEE802.3ab / IEEE802.3z |
| Support VLAN | support IEEE 802.1Q VLAN |
| Maximum package | supports up to 1600 Bytes of packet transmission |
| Network port sequence | automatic identification through and cross the line |
| Ethernet connector | RJ45 interface |
| Power and environmental indicators |  |
| Power | 3W |
| Power supply | +9V~+36V DC |
| Operating temperature | -0°C~+50°C |
| Storage temperature | -50°C~+85°C |
| Working humidity | 0 ~ 95% non-condensing |
| Physical characteristics |  |
| Independent size (mm) | 67L * 75W * 26H |
| Card Type size (mm) | 67Lx75Wx12H |
| Independent weight (kg) | 0.25 |
| Card Type Weight (kg) | 0.05 |
| Mean time between failures (MTBF) | 100,000 hours or more |
| LED indicator | power, Ethernet, optical link, full / half duplex |

LED指示灯