Fiber Media Converter Series

1/3

10/100/1000M Fiber Media Converter

Product Description

TBC-MC3612N-D20 series 10/100/1000M adaptive fiber media converter through 100Base-TX or 1000Base-TX to 100Base-FX photoelectric conversion between the traditional 100M Ethernet or 1000M Fast Ethernet through the Fast Ethernet fiber optic link extended to 120 km range, to meet the long-distance, high-speed , High-bandwidth fast Ethernet working group user needs, the product performance is stable and reliable, designed to meet the Ethernet standards, and anti-lightning protection measures, especially for telecommunications, mobile, China Unicom, safe city, radio and television, railway, military, financial securities, customs, civil aviation, maritime transport, electricity, water conservancy and oil fields and other broadband data network and require high reliability data transmission or the formation of IP data transmission network area, it is the best application equipment for broadband campus network，intelligent broadband district fiber to the floor , Fiber to the home and video surveillance .

Product Features

- Support full duplex or half duplex network communication, and with auto-negotiation capability;

- Supports the transmission of long packets of VLANs;

- Support for IEEE802.1Q protocol switch products to match the use;

- Support LINK LOSS link remote pre-alarm function;

- Automatically adapt to 10Mbps ,100Mbps and 1000Mbps environment, easy to upgrade the network;

- RJ45 port support through or cross automatic recognition function (AUTO-MDIX);

- REMOTE PAUSE function ensures the integrity of packet transmission;

- With broadcast storm protection, balance flow, isolation conflicts and detection errors and other functions;

- Meet the electromagnetic interference standard FCC Part15 Class A and CE Mark safety regulations;

- High quality laser device with good light characteristics, electrical characteristics and temperature stability;

Fiber Media Converter Series

2/3

Product Specification

| Standard Protocol | IEEE 802.3ab, IEEE 802.3Z, IEEE 802.3X, |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Band Width | RJ 45 Port |  | 10/100/1000M |  | Optical Port | 1.25G |  |
| Operation Mode | Full /Half duplex mode,with auto-negotiation capability |  |  |  |  |  |  |
| Connectors | UTP | RJ45 |  |  | Fiber connector | SC/FC/ST/LC |  |
| LED Indicators | 1000M,100M, FX/ACT, TP/ACT , PWR,FDX. |  |  |  |  |  |  |
| UTP Cable | Cat5 UTP （within 100m） |  |  |  |  |  |  |
| Fiber Cable | 9/125μm(the max distance up to 20 -120km) |  |  |  |  |  |  |
|  | 50/125,62.5/125μm(the max distance up to 2km) |  |  |  |  |  |  |
| Power Supply | External power supply |  |  | 5V, 2A |  |  |  |
|  | Internal power supply |  |  | DC-48V, AC110-250V/50Hz |  |  |  |
|  | Power Consumption |  |  | ≤5W |  |  |  |
| Working Environment | Work Temperature |  |  | 0℃~50℃ |  |  |  |
|  | Storage Temperature |  |  | -40℃~70℃ |  |  |  |
|  | Humidity |  |  | 5%~90% non-condensing |  |  |  |
| MTBF (Hours) | >30000 |  |  |  |  |  |  |
| Size and Weight | structure |  |  | Dimensions(mm) |  |  | Weight |
|  | External power supply |  |  | 94mm*70mm*25mm |  |  | 0.4 kg |
|  | Internal power supply |  |  | 140mm*110mm*30mm |  |  | 0.8kg |
|  | Socket Card |  |  | 157mm*128mm*31mm |  |  | 1.0kg |
| Certification | FCC Part 15 , Class A, RoHS and CE Mark |  |  |  |  |  |  |

Fiber Media Converter Series

2/3

LED Indicators

Application

| Name | Definition | Specification |
| --- | --- | --- |
| PWR | Indicator of power supply | ON when the power supply is turned on and in normal working status |
| FX/ACT | Optical interface status indicator | Bright : optic fiber cable is connected well, but no data transmission |
|  |  | Blinking: when receiving data |
| TP/ACT | Ethernet interface status indicator | Bright : twisted pair is connected well, but no data transmission |
|  |  | Blinking: when receiving data |
| 100M | Rate indicator | Bright: The data is being transmitted: 100M |
|  |  | Off: no data is being transmitted |
| 1000M | Rate indicator | Bright: The data is being transmitted: 1000M |
|  |  | Off: no data is being transmitted |
| FDX | Ethernet interface Full / half duplex Signal indicator | Bright: The network interface operates as full duplex |
|  |  | OFF, The network interface operates as half duplex |