Fiber Switch Series

1/2

10/100M 1 fiber port + 4 RJ45 ports fiber switch

Product Description

The TBC-MC3414E-S20 fiber switch can convert 4 independent 10/100Base-TX twisted pair electrical signals and 1 independent 100Base-FX optical signal to each other.It extends the transmission distance of the network from the 100m limit of copper wire to 120km.Through the use of switching technology , storage and forwarding technology to achieve data of the two networks before connecting the media , greatly improving the flexibility of fiber Ethernet set, and reducing the cost of network construction, support dual-fiber multi-mode, dual-fiber single-mode , Single-mode single fiber and other optical fiber transmission.

Product Features

- Storage and forwarding capabilities to support rate adaptation and protocol translation;

- Adaptive connection 10M and 100M network and the network port supports full-duplex/ half-duplex adaptation and traffic control;

- Automatic detection and correction of incorrect poles on each port of a twisted pair;

- Data transfer rate (line speed) is 14,880 frames per second (10M);

- Data transfer rate (line speed) is 14,8800 frames per second (100M);

- The amount of power ≤ 5W;

- Support 12K effective MAC address table; MAC Supports MAC address learning and aging automatically;

- 8 MB packet buffer;

- With broadcast storm protection ;in line with IEEE802.3, IEEE802.3u standards;

- Flow control methods: Full duplex IEEE802.3x standard, Half-duplex using Back-pressure technology;

- 4*10/100M adaptive RJ45 ports, support port auto-flip (AutoMDI / MDIX);Provide 1*100M optical port to meet the requirements of long-distance transmission;

- Support 4 groups of Port VLAN and MTU VLAN functions;Provides port bandwidth control with a minimum granularity of 128kbps;

- Support IEEE802.1p priority protocol mode, the second static port priority Level mode and ToS priority mode ;

- Support for broadcast storm control,Provide fixed IP address settings;

- Support switch system software upgrades, configuration file saving and recovery;

- External 5V1A switching power supply;

- Support physical isolation ;

1 fiber port + 2 RJ45 port Series

2/2

Product Specification

| Standard | IEEE 802.3 10BASE-T Ethernet, IEEE 802.3u 100BASE-TX Fast Ethernet, IEEE 802.3x |  |
| --- | --- | --- |
| Protocol | CSMA/CD |  |
| Data Transmission Rate | Ethernet : 10Mbps(Half-duplex) 20Mbps(Full-duplex) Fast Ethernet : 100Mbps(Half-duplex) 200Mbps(Full-duplex) |  |
| Power Input | 5VDC, 2A |  |
| Work environment | Work Temperature | 0℃~50℃ |
|  | Storage Temperature | -30℃~60℃ |
|  | Humidity | 5%~90% non-condensing |
| Dimensions(mm) | 232mm*142mm*45mm |  |
| Weight | 0.69kg |  |