Fiber Optical Modem Series

1/2

ETH Fiber Optical Modem

Product Description

The function of TBC-6103 series 10/100 Base-T Optical Fiber MODEM is to directly modulate a 10/100Base-T Ethernet data to single-mode or multimode optical fiber, and users can choose data transmission rate from 64Kbps to 2048kbps.

10/100Base-T Optical Fiber MODEM can be used as a user terminal device and E1 optical fiber MODEM connection. The E1 interface of E1 Optical Fiber MODEM can be connected with the Ethernet bridge through SDH and other equipment, and Ethernet bridge can directly monitor no-node Ethernet fiber Modem. It can be used as a relay transmission device for the local network, and is especially suitable for the optical fiber terminal transmission equipment and renting equipment as the base station of the mobile communication network. The equipment is small in size, light in weight, compact in external structure, convenient in installation and maintenance, good in transmission quality and reliable in operation.

Features

- Integrated circuits based on proprietary intellectual property;

- Ethernet interface 10/100M, full / half duplex fully adaptive, support VLAN protocol;

- Ethernet port supports AUTO-MDIX (cross line and straight line adaptation);

- Provides 3 types of loopback functions: local optical loopback (ANA), local Ethernet loopback, and command peer Ethernet loopback;

- Can follow the rate of the remote Ethernet bridge settings, to achieve remote rate adjustment;

- Pseudo-random code test function, easy to open the circuit, that can be used as a E1 BER tester;

- Ethernet monitoring automatic reset function, will not crash;

- Has a fully functional network management capabilities;

- Has a complete warning and status instructions;

- adapt to a variety of power environment-48VDC or 220VAC;

Fiber Optical Modem Series

2/2

Technical Specification

| 10/100Base-T interface | Rate | 10/100M, full duplex, half duplex fully adaptive |
| --- | --- | --- |
|  | Protocol | Support IEEE802.3, IEEE802.1Q (VLAN) |
|  | MAC address table | can learn 4096 MAC address |
|  | Ethernet Total Cache | 64MBits SDRAM |
|  | Physical interface | RJ45 , support AUTO-MDIX (cross line linear adaptive) |
| Optical Interface | Wavelength | Single mode 1310nm / 1550nm Multimode 850nm / 1310nm |
|  | Fiber Type | Dual-fiber bi-directional transmission, single-fiber bi-directional transmission |
|  | Transmit power | -9dBm / -5dBm |
|  | Transmission length | 2km, 20km, 40km, 60km, 80km |
|  | Receiver sensitivity | Better than -36dBm (bit error rate <10-11) |
|  | Optical code | Scramble NRZ code |
|  | Connector | SC / FC / ST(optional) |
| Working environment | Working temperature | 10°C ~ 50°C |
|  | Storage temperature | -40°C ~ 70°C |
|  | Relative Humidity | 5%~95% (non-condensing) |
|  | Non-corrosive and solvent gases, no dust, no strong magnetic interference |  |
| Power supply conditions | DC power supply voltage | DC-48V, tolerance range 36v ~ -72v |
|  | AC supply voltage | AC110 ~ 240V, 50Hz |
|  | Power consumption | ≤ 5W |
| Dimension | 210mm*120mm*30mm(Desktop) |  |