Telephone Multiplexer Series

1/2

4-Channel Telephone Multiplexer

Description

The TCE-DH6804 series Telephone Multiplexer is a point-to-point optical optical transmission equipment developed on the basis of very large-scale integrated circuits. It implements 1*10/100M Ethernet data channel in a compact form and 4-channel voice channel (FXO, FXS), optical multiplexing transmission equipment. With alarm monitoring, functional, high integration, low power consumption, stable performance, easy to use.

Features

- Remote monitoring of the remote device by the local device (unique double-row indicator, upper-row indication and lower-row indication) Integrated circuit based on independent intellectual property;

- Ethernet interface 10M/100M, full / half duplex fully adaptive, support VLAN protocol;

- Provide two kinds of clock: main clock, line clock;

- Provides three kinds of loopback function: The local end of the optical loop, Ethernet local loop, so that the opposite end of the Ethernet ring;

- You can set the auto-rate setting of the remote device commanded by the local device;

- Pseudo-random code test function, easy to open circuit that can be used as a bit error meter;

- Ethernet can support N * 64k rate, the rate is adjustable from 64K-1984K;

- Ethernet monitoring automatic reset function, will not crash;

- Rich Ethernet data prompt function, real-time prompt data communication status;

- Provide 1-4 voice access to support for caller ID and reverse billing functions;

- Support each site put the number function;

- Voice port supports FXO port and FXS port , FXO port and program-controlled switch docking, FXS port and the user's phone connected;

| - Optical interface (single fiber / dual fiber) |  |
| --- | --- |
| Wavelength | Single mode:1310nm / 1550nm; Multimode 850nm / 1310nm |
| Fiber | multimode, single mode |
| Transmit power | 9dBm / -5dBm |
| Transmission length | multi-mode 2 km, single mode 40,60,120 km |
| Receiver sensitivity | better than-36dBm (BER <e-11) |
| Receive dynamic range | > 27dB |
| Connector | SC / FC / ST |

| FXS phone interface |  |
| --- | --- |
| Physical interface | RJ11 |
| Ringing voltage | 75V |
| Ringing frequency | 25Hz |
| Second-line input impedance | 600Ω (off-hook) |
| Return loss | 40dB |

| FXO switch interface |  |  |
| --- | --- | --- |
| Ringing voltage |  | 35V |
| Ringing frequency |  | 17Hz-50Hz |
| Second-line input impedance |  | 600Ω (off-hook) |
| Return loss |  | 40db |
| Ethernet |  |  |
| Physical interface | RJ45 , support Auto-MDIX |  |
| Protocol | Support IEEE802.3 , IEEE802.1Q(VLAN) |  |
| Network Interface Rate | 10 / 100M adaptive |  |
| Working mode | duplex / half duplex adaptive |  |
| MAC Address Table | You can learn 4096 MAC addresses |  |

Telephone Multiplexer Series

2/2

Technical Specification

Application