Telephone Multiplexer Series

1/2

16-Channel Telephone Multiplexer

Description

The TCE-DH6816 series Telephone Multiplexer (4E1 16-Channel FXO / FXS) is a point-to-point optical transmission equipment developed on the basis of the ultra-large-scale PDH optical fiber transmission developed by our company. Has a sound alarm function, optional 1 to 4 100M wire speed Ethernet interface and 1 to 4 serial data interface. Device can be built-in SNMP module, stand-alone SNMP management. Electrical equipment for all-digital circuit, the whole work reliable, stable, low power consumption, high integration, small size, easy to install and maintain.

Features

- Integrated circuit based on independent intellectual property rights;

- The whole machine is designed for veneer;

- Can monitor the alarm status of the peer;

- E1 interface complies with G.703 recommendations, using fully digital clock recovery and smooth phase lock technology;

- Provide two expansion interfaces, which can expand 1~4 asynchronous data, such as RS232 / RS485 / RS422 / Manchester code, etc .; or 1~8 switches and 1~4 E1(Therefore, up to 8 E1 channels on this device), 1~4 voice, or two/four-wire audio;

- Ethernet interface 10M / 100M, full / half duplex fully adaptive, VLAN support;

- Each Ethernet port supports Auto-MDIX (cross / straight connection adaptation);

- 1-16 voice access, support for caller ID function;

- Support each site's mutual release number function;

- Voice port support FXO port and FXS port, FXO port and program-controlled switch docking, FXS port and the user's phone connected;

- Desktop equipment can support independent SNMP network management, and support optical network management level connected (optional);

- Can detect the loss of optical signals for the remote device off or fiber fails;

- Can monitor the local temperature and the local voltage;

- Can monitor the voltage and temperature of the remote device and set the temperature alarm threshold;

- A variety of power options: AC220V, DC-48V / DC24V, etc .;

- No relay transmission distance greater than 2 ~ 120 km.

Telephone Multiplexer Series

2/2

Technical Specification

| Optical interface |  |
| --- | --- |
| Wavelength | single mode 1310nm / 1550nm multimode 850nm / 1310nm |
| Fiber | multimode, single mode |
| Transmit power | -9dBm / -5dBm |
| Transmission length | 0-50 km / 120 km |
| Receiving sensitivity | better than-36dBm (bit error rate <10-11) |
| Optical code | Scramble NRZ code |
| Connector | SC / FC / ST |
| E1 interface |  |
| 4 E1 interfaces are in line with G.703 recommendations |  |
| Rate | 2048Kb / s ± 50ppm |
| Pattern | HDB3 |
| Impedance | unbalanced 75W |
| Jitter characteristics | to meet the G.742, G.823 recommendations |
| Input port allows attenuation | 0 ~ 6dBm |
| Physical Interface | BNC |
| Ethernet interface (10 / 100Base-T) |  |
| Bandwidth | line speed 100M |
| Rate | 10 / 100M adaptive, full / half duplex adaptive |
| Protocol | Supports IEEE 802.3, IEEE 802.1Q (VLAN) |
| MAC Address Table | learn 4096 MAC addresses |
| Physical interface | RJ45 , support Auto-MDIX (cross / straight line adaptive) |
| FXS phone port |  |
| Ringing voltage | 75V |
| Ringing frequency | 25HZ |
| Second-line input impedance | 600Ω (off-hook) |
| Return loss | 40 dB |
| FXO Interface |  |
| Ringing detection voltage | 35V |
| Ringing detection frequency | 17HZ-60HZ |
| Second-line input impedance | 600Ω (off-hook) |
| Return loss | 40 dB |