Fiber  Switch Series

1/3

10/100/1000M 2 fiber ports +8 RJ45 ports Switch

Product Description

The TBC-MC3728E series Ethernet optical switch has 2*1000M optical port and 8*10/100/1000M adaptive RJ45 interface, users can choose according to the needs of different 1*9 fixed or hot-swappable SFP Optical modules, such as multi / single mode, single / dual fiber, single mode 550m ~ 120Km and so on. Can realize the Ethernet optical, seamless connection between electrical signals, cascading. Switching chip using the latest American IC, stable performance, good quality, easy to use, with high cost, suitable for small fiber Ethernet and fiber optic Ethernet networks with special needs.

Product Features

- 2  SFP / 1*9 Optical port and 8*10/100/1000M RJ45 ports (optional);

- Optical interface supports three kinds of fiber multimode dual fiber, single mode dual fiber, single mode single fiber ;

- RJ45 electrical port can adapt to 10/100/1000M, full / half duplex mode;

- RJ45 port supports straight or cross automatic identification (AUTO-MDIX);

- Optical port supports 1000M full duplex mode;

- Support flow control, can prevent broadcast storm;

- support up to 1632 bytes long packet;

- Small power consumption (10W), less heat, good stability;

- SFP module support hot-swappable.

LED Indicator

| Name | Definition | Description |
| --- | --- | --- |
| PWR | Power Indicator | ON when the power supply is turned on and in normal working status |
| FX1/LINK FX2/LINK | Optical interface status indicator | Bright :fiber link connectivity, no data transmission; Blinking: fiber link is connected, data is being transmitted |
| LINK | Rate indicator | Bright :twisted pair link is connected |
|  |  | Blinking: twisted pair link is connected, the data is being transmitted |
| SPD | Network port mode indicator | Bright: fiber optic transceivers work in 100M; network port full duplex mode of work |

Fiber  Switch Series

2/3

Product Specification

| Standard Protocol |  | IEEE802.3u/z/ab,10Base-T,100Base-TX,1000Base-X |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rate |  | - 1Gbps |  |  |  |  |
| Operation Mode |  | Full /Half duplex mode |  |  |  |  |
| Optical interface |  | SC /ST or FC / LC (default SC interface) |  |  |  |  |
| Physical interface |  | RJ45 port |  |  |  |  |
| Multi mode | Suitable for optical fiber | 62.5/125,50/125,100/140µm |  |  |  |  |
|  | Transmission distance | 550m 2Km |  |  |  |  |
|  | Transmit power | >-20~-14dBm |  |  |  |  |
|  | wavelength | 850nm//1310nm |  |  |  |  |
|  | Receiving sensitivity | <-31dBm |  |  |  |  |
| Single mode | Suitable for optical fiber | 9/125、8.3/125、8.7/125 or 10/125µm |  |  |  |  |
|  | Transmission distance | 25km | 40km | 60km | 80km | 120km |
|  | Transmit power | -12~-8dBm | -8~-3dBm | -3~0dBm | -3~1dBm | -3~2dBm |
|  | wavelength | 1310nm | 1310nm | 1490nm /1550nm | 1490nm /1550nm | 1490nm /1550nm |
|  | Receiving sensitivity | < -32dBm | < -34dBm | < -34dBm | < -38dBm | < -38dBm |
| Work Temperature |  | -20 ℃ ~ +65 ℃ (Commercial ) - 40 ℃ ~ +85 ℃ (Industrial ) |  |  |  |  |
| Power Supply |  | AC100-260V 48VDC (Internal power supply),DC12V 24V 5V (External power supply) |  |  |  |  |
| Storage Temperature |  | -40℃~+90℃ |  |  |  |  |
| Relative humidity |  | 5%~90% non-condensing |  |  |  |  |
| MTBF (Hours) |  | >50000 |  |  |  |  |
| Certification |  | FCC Part 15 , Class A, RoHS and CE Mark SGS |  |  |  |  |

Packing list

2 fiber ports & 8 RJ45 port fiber switch

power cable

user's Guide

Warranty Card