Fiber Switch Series

1/2

10/100/1000M 2 GE fiber port & 8 GE RJ45 ports Fiber Switch

Product Description

2*10/100/1000M Optical ports + 8*10/100/1000M adaptive RJ45 ports Ethernet optical switch , the user can choose different 1*9 fixed or hot-swappable SFP Modules, such as multi/single mode, single/dual fiber, single mode 550m ~ 120Km and so on. Can realize the Ethernet optical, seamless connection between electrical signals, cascading. Switching chip using the latest American IC, stable performance, good quality, easy to use, with high cost, suitable for small fiber Ethernet and fiber optic Ethernet networks with special needs.

Product Features

- Provide 2 *SFP /1*9 module optical ports and 8*10/100/1000M RJ45 RJ45 ports (optional);

- Optical interface supports multimode dual fiber, single mode dual fiber, single mode single fiber ;

- RJ45 port can adapt to 10/100/1000M, full / half duplex mode;

- RJ45 port supports straight or cross automatic identification (AUTO-MDIX);

- Optical port supports 1000M full duplex mode;

- Support flow control, can prevent broadcast storm;

- Support up to 1632 bytes long packet;

- Small power consumption (10W), less heat, good stability;

- SFP optical module supports hot swap.

LED Indicator

| Name | Definition | Description |
| --- | --- | --- |
| PWR | Power Indicator | ON when the power supply is turned on and in normal working status; |
| FX1/LINK FX2/LINK | Optical interface status indicator | Bright : optic fiber cable is connected well, but no data transmission; Blinking: when receiving data; |
| LINK | Rate indicator | Bright: twisted pair link is connected; Blinking, twisted pair link is connected, data is being transmitted |
| SPD | RJ45 Port status indicator | Bright: fiber transceivers operate at 1000M The network port works in full duplex mode |

Fiber Switch Series

2/2

Product Specification

| Standard Protocol | IEEE 802.3u, IEEE 802.3Z, IEEE 802.3ab,10Base-T, 100Base-TX,1000Base-X |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band Width | RJ 45 Port : 8*10/100/1000M |  |  | Optical Port: 1.25G |  |  |
| Operation Mode | Full /Half duplex mode,with full duplex / half duplex auto-negotiation |  |  |  |  |  |
| Connectors | UTP: RJ-45 |  |  | Fiber connector : SC/FC/ST/LC |  |  |
| Multi mode | Applicable optical fiber: 62.5/125,50/125,100/140μm Transmission distance: 550m 2Km Transmit power > -20 ~ -14dBm Wavelength :850nm / 1310nm Receiver sensitivity <-31dBm |  |  |  |  |  |
| Single mode | Applicable optical fiber: 9/125,8.3/125,8.7/125 ,10/125µm |  |  |  |  |  |
|  | Transmission distance | 25km | 40km | 60km | 80km | 120km |
|  | Transmit power | -12~-8dBm | -8~-3dBm | -3~0dBm | -3~1dBm | -3~2dBm |
|  | Wavelength | 1310nm | 1310nm | 1490/1550nm | 1490/1550nm | 1490/1550nm |
|  | Receiver sensitivity | < -32dBm | < -34dBm | < -34dBm | < -38dBm | < -38dBm |
| Power Supply | External Power supply |  |  | DC 12V 24V 5V |  |  |
|  | Internal Power Supply |  |  | AC 100-260V 48VDC |  |  |
| Environmental Parameters | Work Temperature |  |  | -20℃~+65℃(Commercial) -40℃~+85℃(Industrial ) |  |  |
|  | Storage Temperature |  |  | -40℃~+90℃ |  |  |
|  | Humidity |  |  | 5%~90% non-condensing |  |  |
| MTBF (Hours) | >50000 |  |  |  |  |  |
| Certification | FCC Part 15 , Class A, RoHS and CE Mark,SGS |  |  |  |  |  |

Packing list

- 2*10/100/1000M Optical ports + 8*10/100/1000M adaptive RJ45 ports Ethernet optical switch

- Power Adapter

- User's Guide

- Warranty Card

http://sztincam.en.alibaba.com