PDH Multiplexer Series

1/2

16E1+ ETH PDH Multiplexer

Product Description

The TBC-6480U1FE Series 16E1 +100M optical transceiver is a point-to-point optical transmission equipment developed on the basis of the ultra-large-scale PDH optical fiber transmission developed by our company. It provides 1 linear 10/100M Ethernet channel. It has a sound alarm function, optional 1-Channel digital business telephone . Electrical equipment for all-digital circuit, the whole work reliable, stable, low power consumption, high integration, small size, easy to install and maintain. Equipment to meet the needs of telecom operators and private network users, in line with telecommunications standards. Suitable for power, postal services, banking and China Unicom, mobile telecommunications and other user needs, in line with the standards used by telecom operators.

- Product Features

- Provide 16 E1 + 10/100BaseT;

- Ethernet interface speed 10M/100M adaptive, support VLAN protocol;

- E1 port with digital clock recovery circuit and digital platform smooth phase lock circuit;

- Provide command E1 loopback function, optical path local loop function;

- Complete power supply, link and alarm indication function, the local device can monitor the current status of the remote device in real time;

- Provide branch alarm and sound alarm shield;

- Provide 1-Channel business telephone interface(optional);

- Dual fiber bidirectional and single fiber bidirectional models are optional;

- Optical interface optional SC/PC, FC/PC,or SFP slot;

- Conforms to carrier-class operation requirements, mean time between failures (MTBF) more than 100,000 hours;

- High reliability, providing 19 "rack mount,desktop,wall,card ,at the same time with AC and DC power supply;

- Power can choose to AC 220V, DC -48V,24V,12V,5V, the whole power consumption <5W; Provide SNMP network management functions;Support customization and OEM .

PDH Multiplexer Series

2/2

Product Parameter

| Optical Interface Features | Wavelength | single mode 1310nm/1550nm,Multi mode 850nm/1310nm |
| --- | --- | --- |
|  | Number of optical fiber | dual-core, single-core |
|  | Transmission power | -9dBm / -5dBm |
|  | Transmission distance | Multi mode 0~2km Single mode 40KM,60KM,120KM |
|  | Receiver sensitivity | ≤-36dBm (BER ≤ 10-11) |
|  | Receive dynamic range | > 27dB |
|  | Connector | FC / SC / ST optional |
| E1 interface Features | Interface code | HDB3 |
|  | Interface code speed | 2.048Mbit / s ± 50ppm |
|  | Interface Features | ITU-T G.703 |
|  | Interface Type | BNC, RJ45 |
|  | Interface impedance | 75Ω unbalanced / 120Ω balanced |
| Ethernet interface features | Interface Rate | 10/100M adaptive |
|  | Interface Protocol | IEEE802.3,IEEE802.1Q(VLAN) |
|  | Physical interface | RJ45,Support Auto-MDIX |
|  | Operate mode | full / half duplex adaptive |
|  | MAC Address Table | can learn 4096 MAC addresses |
| Power | Power supply | AC 220V, DC -48V,24V,12V,5V |
|  | Power consumption | ＜5W |
| Working environment | Working temperature | 0°C ~ 50°C |
|  | Humidity | 5%~95% (non-condensing) |
|  | Storage temperature | -40°C ~ 70°C |

Application