Telephone Multiplexer Series

1/3

16-Channel Telephone Multiplexer

Description

The TBC-6816U series 16-Channel Telephone Multiplexer is a new generation of highly integrated single board PCM group multiplexing equipment based on proprietary software and thick film technology. The equipment can simultaneously transmit 16-channel  telephone interfaces through optical fiber and transmit E1 signals, Audio signals, data signals, etc. (The above interfaces are based on customer requirements flexible customization, not the standard configuration). The Telephone Multiplexer Series equipment is mainly used for transmission through the network, telephone and program-controlled switches to achieve the connection. Equipment is widely used in: Internet cafes, small and medium-sized companies, highways and temporary construction sites need to address the needs of the rapid telephone and data services, as well as the ongoing construction of the village through telephone access and so on.

Features

- Industrial design, SMT process ;

- All digital optical transmission platform, with the platform multi-service flexible configuration;

- Perfect alarm output: can provide real-time business alarm, optical error detection, communication interface self-test instructions;

- High stability: the selection of advanced and reliable electronic components and optical components at home and abroad, in the ambient temperature -20 ℃ ~ +65 ℃, operating humidity 0 ~ 95% (non condensing) of the state of work;

- Easy installation, no configuration required;

| Optical interface |  |
| --- | --- |
| Physical interface | FC / PC, SC / PC, ST/ PC |
| Transmission wavelength | 850nm / 1310nm / 1550nm |
| Transmission distance | 0~20Km (single mode fiber, standard type) 0~60Km (single mode fiber, long distance type) 0~120km (single-mode optical fiber, long-range type) 0~2km (multimode optical fiber, standard type) |
| Transmitter power | better than - 8dB |
| Receiver sensitivity | better than -23dB |
| Optical dynamic range | 12dB |
| Optical maximum chain loss allowed | 12dB |

Telephone Multiplexer Series

2/3

Technical Specification

| FXS phone interface |  |
| --- | --- |
| Physical interface | RJ11 |
| Ringing voltage | 75V |
| Ringing frequency | 25Hz |
| Second-line input impedance | 600Ω (off-hook) |
| Return loss | 40dB |

| FXO switch interface |  |
| --- | --- |
| Ringing voltage | 35V |
| Ringing frequency | 17Hz-50Hz |
| Second-line input impedance | 600Ω (off-hook) |
| Return loss | 40db |

| Magnet interface |  |
| --- | --- |
| Physical interface | RJ45 |
| Audio impedance | -20dB |
| Power supply rejection ratio | -30dB |
| Idle noise | -65dB |
| 2/4 line E & M audio |  |
| Physical interface | RJ45 |
| A D Gain | 0dB |
| D A gain | -3.5dB |
| 2/4 line input impedance | 600Ω |
| Return loss | 20 dB |

| Data interface |  |
| --- | --- |
| Physical Interface | Standard Industrial Crimp Terminal (Phoenix Terminal) |
| Data direction | reverse or bidirectional RS232 / 485 |
| Data bandwidth | 0-38.4Kbps |
| Standard | RS-485/232 |
| Bit error rate | <10-9 |
| Ethernet |  |
| Physical interface | RJ45 |
| Ethernet protocol | Comply with IEEE802.310 / 100Base-T |
| Network Interface Rate | 10 / 100M adaptive |
| Network transmission rate | 100M |
| Working conditions |  |
| Operating temperature | -20 ℃ ~ +65 ℃ |
| Working humidity | 0 ~ 95% (no condensation) |
| Power consumption | 10-50W @ DC 24V / -48V or AC: 220V |
| Non-corrosive and solvent gases, no dust, no strong magnetic interference |  |

Telephone Multiplexer Series

3/3

Technical Specification