- TBC-STR1800

- Ceiling wireless AP

▼Product description

TBC-STR1800 is a high-performance, high-speed indoor 11ax dual-band ceiling AP, with a maximum access rate of 574Mbps for 2.4G and a maximum access rate of 1201Mbps for 5G.

▼Picture

▼Features

- It has better transmission quality and higher transmission rate, and the concurrent rate of the whole machine can reach up to 1775Mbps.

- Support hardware watchdog function, automatic recovery of abnormal equipment, maintenance-free。

- Elegant appearance design, port hidden shell, beautiful wiring.

- Support 48V PoE, DC 12V two power supply modes.

- ▼Product parameters

| Hardware Specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Model | TBC-STR1800 |  |  |  |  |
| CPU | MT7621AT+MT7905N+MT7975DN |  |  |  |  |
| Flash | SPI NOR 16MB |  |  |  |  |
| Memory | 256M |  |  |  |  |
| 2.4G Working frequency | 2.4GHz – 2.484GHz |  |  |  |  |
| 2.4G WIFI Transfer Protocol | 802.11 b / g / n /ac /ax |  |  |  |  |
| 5G Working frequency | 5.150GHz～5.850GHz |  |  |  |  |
| 5G WIFI Transfer Protocol | 802.11 a / n / ac /ax |  |  |  |  |
| Built-in antenna 2 .4G | Antenna × 2PCS, gain: 4.6DBi |  |  |  |  |
| Built-in antenna 5.8G | Antenna × 2PCS Gain: 4DBi |  |  |  |  |
| Top speed | 2.4G provides a maximum access rate of 574Mbps, and 5G provides a maximum access rate of 1201Mbps |  |  |  |  |
| 2.4G wireless power | 802.11b | 11M | 20±2dBm | 1M | 20±2dBm |
|  | 802.11g | 54M | 19±2dBm | 6M | 20±2dBm |
|  | 802.11n HT20 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11n HT40 | MCS7 | 17±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT20 | MCS9 | 17±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT40 | MCS11 | 16±2dBm | MCS0 | 18±2dBm |
| 5G wireless power | 802.11a | 54M | 18±2dBm | 6M | 20±2dBm |
|  | 802.11n HT20 | MCS7 | 17±2dBm | MCS0 | 19±2dBm |
|  | 802.11n HT40 | MCS7 | 16±2dBm | MCS0 | 18±2dBm |
|  | 802.11ac HT80 | MCS9 | 15±2dBm | MCS0 | 17±2dBm |
|  | 802.11ax HT80 | MCS11 | 14±2dBm | MCS0 | 16±2dBm |
| 2.4G receiving sensitivity | 802.11b | 11M | -85dBm | 1M | -92dBm |
|  | 802.11g | 54M | -72dBm | 6M | -90dBm |
|  | 802.11n HT20 | MCS7 | -70dBm | MCS0 | -88dBm |
|  | 802.11n HT40 | MCS7 | -68dBm | MCS0 | -86dBm |
|  | 802.11ax HT20 | MCS11 | -60dBm | MCS0 | -85dBm |
|  | 802.11ax HT40 | MCS11 | -56dBm | MCS0 | -85dBm |
| 5G reception sensitivity | 802.11a | 54M | -72dBm | 6M | -92dBm |
|  | 802.11n HT20 | MCS7 | -70dBm | MCS0 | -90dBm |
|  | 802.11n HT40 | MCS7 | -68dBm | MCS0 | -88dBm |
|  | 802.11ac HT80 | MCS9 | -58dBm | MCS0 | -85dBm |
|  | 802.11ax HT80 | MCS11 | -55dBm | MCS0 | -84dBm |
| 2.4G EVM | 802.11b:≤-10 dB；802.11g:≤-25 dB；802.11n:≤-28dB ; 802.11ax: ≤-35 dB |  |  |  |  |
| 5G EVM | 802.11a:≤-25 dB； 802.11n:≤-28 dB；802.11ac:≤-32 dB ; 802.11ax:≤-35 dB |  |  |  |  |
| frequency offset（ppm） | ±20ppm |  |  |  |  |
| interface (WAN) | 1 10/100/1000M adaptive WAN port, support 48V POE power supply |  |  |  |  |
| interface（LAN） | 1 10/100/1000M adaptive LAN port |  |  |  |  |
| POE | 802.11af |  |  |  |  |
| DC | 12V 1.5A |  |  |  |  |
| Reset | reset button (press and hold for 6-10 seconds to restore the factory default configuration) |  |  |  |  |
| Status Indicator | WAN port light, LAN port light, three-color light (three different colors: sys-red, 2.4G-green, 5.8G-blue) |  |  |  |  |
| Maximum power consumption | < 15W |  |  |  |  |
| Product size | 188mm * 188 mm * 35 mm |  |  |  |  |
| working environment | Normal operating temperature: -20°C to 45°C; Storage temperature: -40°C to 70°C; Humidity: 5%～95% (non-condensing) |  |  |  |  |