TBC-PTR300 300M

Wireless panel AP

▼Product description

This is a high-performance dual-band 802.11N in-wall AP that supports intelligent channel analysis, automatic optimal channel selection, and can support a total bandwidth of up to 300Mbps . This product supports gateway, AP and other modes, which can provide users with a variety of networking methods, making user application scenarios more extensive.

▼Product picture

▼Product feature

- Support hardware watchdog function, maintenance-free；

- Button design: turn off the light and turn off the WiFi function to ensure that the user is in a comfortable environment；

- Integrated PoE (IEEE 802.3af PoE) to reduce deployment overhead；

- Internal antenna is factory optimized for maximum RF performance.

- ▼Product parameters

| Specification |  |
| --- | --- |
| CPU | MT7628DAN |
| Flash | 8M |
| DDR2 | 64M |
| 2.4G Working frequency | 2.4GHz – 2.484GHz |
| 2.4G WIFI Transfer Protocol | 802.11 b / g / n |
| Built-in antenna 2 .4G | Omnidirectional antenna 2pcs, each gain: 2dBi |
| SSID hide | Support |
| Top speed | 300Mbps |
| Maximum support for wireless access | 50+ |
| Interface (WAN) | 1 10/100M adaptive WAN port, support 48V POE power supply |
| Interface （LAN） | Interface (LAN) 1 10/100M adaptive LAN port |
| POE | 802.3af |
| Reset | reset key (short press once to turn off the light, short press twice in a row to turn off the light and WIFI, long press for 10 seconds to restore the factory default configuration) |
| Status Indicator | system lights |
| Maximum power consumption | <8W |
| ESD test- I | air discharge:±4KV |
| Surge test- I | common mode：±2KV |
| Product size | 87mm*86mm*35.8mm |
| working environment | Normal operating temperature: -20°C to 40°C; Extreme operating temperature -30°C to 45°C; Storage temperature: -40°C to 70°C; Humidity: 5%～95% (non-condensing) |

- ▼Software function

| Support mode | Gateway mode: The device connects to the WAN through the WAN port and then uses static IP, DHCP or PPPOE to access the Internet |
| --- | --- |
|  | AP mode: The device covers the wireless network signal to other clients or devices, and uses an Ethernet cable to connect to other routers. into the WAN |
| wireless function | SSID broadcast |
|  | Number of SSIDs: 4 (2.4GHz), support Chinese SSID |
|  | Hide SSID feature |
|  | Wireless authentication method: OPEN, WPA/WPA2-PSK |
|  | Channel and channel bandwidth can be set |
|  | Wireless MAC address filtering: support whitelist (100) |
|  | VLAN ID: VLAN-ID range must be 3~4094, 0 means not enabled |
|  | Spectrum Navigation (5G Priority) |
|  | WiFi timer off function |
|  | User isolation: isolation between wireless networks, AP internal isolation |
|  | Transmit power setting |
|  | The number of wireless clients is limited, and the maximum number of users supported is 100 |
|  | Provides the WIFI analysis function of the current environment |
| Network features | IPv4 address: supports static addresses, supports obtaining addresses from management servers, and obtaining addresses from network management |
|  | DHCP server: either a built-in DHCP server or an external DHCP server |
|  | Cloud platform server |
|  | Firewall function |
|  | Layer 2 and Layer 3 roaming: By working with the AC controller, wireless users can roam seamlessly between Layer 2 and Layer 3 networks |
|  | PPPOE function: Support PPPOE dial function |
| Device management | Backup configuration information |
|  | Restoring configuration information |
|  | reset |
|  | Restart includes timed restart and immediate restart |
|  | change Password |
|  | Firmware upgrade, optional whether to restore factory settings after the upgrade is complete |
|  | Time management including system display time and time synchronization |
|  | System log |
|  | Fat and thin AP switching function |

- ▼Product picture

- Product Instruction Chart

- Product outline