- TBC-APE2521AB

- 5.8G Wireless bridge

▼Product description

TBC-APE2521AB is a 5G_11AC high-performance, high-speed outdoor long-distance wireless bridge. It has the characteristics of long transmission distance, strong penetrating ability, and strong anti-interference ability.

▼Product picture

▼Product feature

- Support intelligent signal road analysis, automatic selection of the highest quality signal road, and the maximum rate is 900Mbps.

- The high-power and high-gain directional antenna design is adopted to make the transmission distance of the product longer and the coverage area wider.

- Support 48V POE or 24V POE (choose one of the two), DC 12V two power supply modes.

- Using a dedicated TVS protection device to improve the stability of the product in harsh environments.

- ▼Product parameter

| Specifications |  |
| --- | --- |
| Product name | TBC-APE2521AB |
| CPU | MT7628DAN+MT7613BEN |
| Flash | SPI NOR 8MB |
| DDR2 | 64MB |
| 5G Working frequency | 5.150GHz～5.850GHz |
| 5G WIFI Transfer Protocol | 802.11 a / n /ac |
| Directional 5G antenna | Gain：14dbi |
| SSID hidden | Support |
| Highest rate | 900Mbps |
| Maximum support for wireless access | 50+ |
| Maximum transmit power | < 27dBm |
| Port (WAN) | 1个10/100M Adaptive WAN port, support POE power supply |
| Port（LAN） | 1个10/100M Adaptive LAN port |
| POE | 48V 0.5A or 24V 0.5A （choose one） |
| DC | 12V 1A |
| Button | reset button (long press for 6-10 seconds to restore the factory default configuration, short press for 1 second to initiate pairing) |
|  | DIP switch |
| Status Indicator | PWR light, WAN port light, LAN port light, 5G WIFI light, signal strength light*4 |
| Waterproof performance | IP65 |
| ESD test | ±6KV |
| Surge test | Signal difference±2KV |
| Maximum power consumption | < 8W |
| Product size | 230mm*75mm*55mm |
| working environment | Normal working temperature: -30°C to 55°C; Maximum operating temperature: -35°C to 75°C; Storage temperature: 0°C to 70°C; Humidity: 5%～95% (non-condensing) |

- ▼Software function

| Operation mode | AP mode: The device covers the wireless network signal to other client terminals and devices, and uses Ethernet to connect to other routers to access the WAN。 |
| --- | --- |
|  | Universal relay: The device is connected to other wireless AP devices through wireless signals, and the wireless network signals of this device are covered by other client terminals and devices |
| Wireless function | SSID broadcast |
|  | Number of SSID: 4, support Chinese SSID |
|  | Hide SSID function |
|  | Wireless encryption：OPEN、WPA、WPA-PSK、WPA2-PSK |
|  | VLAN ID: The range must be 3-4094, 0 means not enabled. |
|  | Wireless MAC address filtering: support whitelist (64) |
|  | Relay function |
|  | WiFi Timing off function |
|  | User isolation: isolation between wireless networks, isolation within APs |
|  | Country selection |
|  | 5G mode: 11AN or 11AC |
|  | Transmit power setting |
|  | Short GI on or off |
|  | The number of wireless clients is limited, and the maximum number of users is 64 |
| Internet function | Set IP address statically or obtain IP address dynamically |
|  | DHCP server |
|  | Cloud platform server |
| Equipment management | Backup configuration information |
|  | Restore configuration information |
|  | Reset |
|  | Restart includes scheduled restart and immediate restart |
|  | Firmware upgrade |
|  | Time management includes system display time and time synchronization |
|  | System log |
|  | Support local unified management function |